This is the kit to upgrade a MESA BIOS. Included are a ROMDOS BIOS

and a standard BIOS image. These image files are called ROMDOS.128 and

ROMBIOS.128. The flash update program is called XXXXWBIO.EXE where XXXX is

the MESA card name. Two batch files (ROMDOS.BAT and ROMBIOS.BAT) use the

flash update program to write a ROMDOS or standard BIOS. These batch files

expect the BIOS image file and the flash update program to be located on

the default drive.

Before upgrading the BIOS you must make sure that no memory managers

are running or the upgrade may fail! This includes HIMEM.SYS, EMM386.EXE

or any similar programs. It is also a good idea to remove any TSRs from the

CONFIG.SYS or AUTOEXEC.BAT files.

The easiest way to upgrade the BIOS is to copy the contents of the

UPGRADE.EXE file to a floppy. Executing the UPGRADE.EXE program will unpack

the files needed. Then the command ROMBIOS (for a normal ROMBIOS) or ROMDOS

(for a ROMDOS BIOS) can be issued with the floppy as the default drive:

A:

A:\ROMDOS

The flash update program will read in the image file, and ask you

whether to write the BIOS. If you say no at this point, the upgrade is

aborted. After Replying with a Y to the question the flash update program

starts re-writing the BIOS. The speaker will beep and the LED (if available)

will blink as the program writes the BIOS. DO NOT reset or power down the

CPU when the speaker is beeping or the BIOS upgrade will fail, and you will

will have to physically replace the BIOS chip. If the flash write program

fails, you are probably up the well known creek, but you can retry the

procedure as long as you don't reset the machine.

Once the BIOS is re-written, you need to reset the CPU to use the

new BIOS. This means a full reset, a warm start (CNTL-ALT-DEL) will not work.

##################################################################################